# Recording Universal Robot Manipulator State Via the RTDE on a Raspberry Pi

Version 1.0

15 August, 2025

Last edited by:  Jeremy Marvel (jeremy.marvel@nist.gov)

Guide is available at https://www.universal-robots.com/articles/ur/interface-communication/real-time-data-exchange-rtde-guide/

Download latest release from https://www.github.com/UniversalRobots/RTDE_Python_Client_Library/releases

Installation of the Python library:

python3 -m pip config set global.break-system-packages true

python3 -m pip install ./rtde-<version>-release.zip



# @brief Launch and run in a new thread the UR RTDE recorder

#

# ======================================================================

def rtde_recorder():

global running_

global recording_

global timestr_

global UR3Rec_

verbose = True

host = "169.254.152.41"   #! TODO:  Replace with IP address of UR controller

port = 30004

freq = 125

binary = False

if verbose:

logging.basicConfig(level=logging.INFO)

conf = rtde_config.ConfigFile("logger.xml")

output_names, output_types = conf.get_recipe("out")

#! Connect to the robot controller

con = rtde.RTDE(host, port)

con.connect()

#! Get controller version

con.get_controller_version()

#! Set up recipes

if not con.send_output_setup(output_names, output_types, frequency=freq):

logging.error("Unable to configure output")

sys.exit()

#! Start data synchronization

if not con.send_start():

logging.error("Unable to start synchronization")

sys.exit()

writeModes = "wb" if binary else "w"

while running_:

if recording_:

rtdefilename = timestr_ + "_rtde.csv"

rtdetimefile = timestr_ + "_rtde_timestamps.csv"

with open (rtdetimefile, writeModes) as rtdetime:

with open(rtdefilename, writeModes) as csvfile:

writer = None

UR3Rec_ = True

if binary:

writer = csv_binary_writer.CSVBinaryWriter(csvfile, output_names, output_types)

else:

writer = csv_writer.CSVWriter(csvfile, output_names, output_types)

print ("Recording UR3 state data")

writer.writeheader()

#! Loop until instructed to stop by the main program

while recording_:

try:

#state = con.receive_buffered(binary)

state = con.receive(binary)               #! Only get the most recent message

if state is not None:

timstr = datetime.datetime.now()

writer.writerow(state)

rtdetime.writerow(timestr)

except rtde.RTDEException:

con.disconnect()

sys.exit()

UR3Rec_ = False

print("Finished writing UR3 log file")

# with open(rtdefilename, writeModes) as csvfile

#! with open() automatically closes the file when completed

# with open (rtdetimefile, writeModes) as rtdetime:

print ("Finalizing UR3 timestamp log")

# while running_

#! Close the connection to the robot controller

con.send_pause()

con.disconnect()

# ======================================================================

