Calibration

| Sensor | Intrinsic Calibration | Extrinsic Calibration |
| --- | --- | --- |
| Camcorder | N/A | Coordinate space registration using an artifact with OptiTrack markers. Sensor uses a custom routine to track an object in its intrinsic coordinate system, while the OptiTrack tracks the pose of the sensor and the artifact. Calculate the transformation from the sensor’s coordinate space to the OptiTrack coordinate space using the NIST registration method. |
| Intel RealSense | Pre-calibrated | Coordinate space registration using an artifact with OptiTrack markers. Sensor uses a custom routine to track an object in its intrinsic coordinate system, while the OptiTrack tracks the pose of the sensor and the artifact. Calculate the transformation from the sensor’s coordinate space to the OptiTrack coordinate space using the NIST registration method. |
| Audio | N/A | OptitTrack marker |
| Captury MoCap | Built-in setup routine per camera | Built-in setup routine between cameras and with the measurement space |
| OptiTrack MoCap | Pre-calibrated | N/A; Built-in setup routine with the measurement space. |
| Vicon MoCap | Built-in setup routine per camera | Built-in setup routine between cameras |
| UR Teach Pendant | Occasional confirmation of robot pose on startup | N/A currently (Probably could put on OptiTrack markers) |
| Forge/OS Teach Pendant | N/A | N/A currently (Probably could put on OptiTrack markers) |
| UR UR3e robot | DH parameters; built-in calibration routine at startup | Intrinsic design of the testbed mounting points |
| Robotiq robotic gripper | Built-in calibration routine at startup | Manual TCP offset on teach pendant |
| Control collar | N/A | N/A currently |

Registration

| Sensor | Data Format | Spatial Registration | Temporal Synchronization |
| --- | --- | --- | --- |
| Camcorder | AVI video and embedded 2-channel audio | Attached OptiTrack marker | Time code generator |
| Intel RealSense |  | Attached OptiTrack marker |  |
| Audio | WAV | Attached OptiTrack marker | Time code generator |
| Captury MoCap | AVI video and tracking data | Modification of the calibration target to include MoCap markers to register with the OptiTrack and/or Vicon |  |
| OptiTrack MoCap | Tracking data |  |  |
| Vicon MoCap | Tracking data |  |  |
| UR Teach Pendant | AVI video |  |  |
| Forge/OS Teach Pendant | AVI video |  |  |
| UR UR3e robot | RTDE data stream |  |  |
| Robotiq robotic gripper |  | Physically attached to the UR3e tool flange |  |
| Control collar |  | Physically attached to the Robotiq gripper |  |