# Configuring Digital Inputs For Enabling Freedrive on Universal Robots Controllers

Version 1.0

7  January, 2025

Last edited by:  Jeremy Marvel (jeremy.marvel@nist.gov)

CB3 Series Controllers

From the main menu, press the “Program Robot” button

Select the “Installation” tab on the new window

Select “I/O Setup” on the left menu.

Choose a digital input to be used as a trigger for enabling freedrive.  Select that input on the left-hand side of the window.

Set the “Action” to “Freedrive” from the drop-down menu

e-Series Controllers

Under the “Installation” tab.

Left hand menu, “General,” select “I/O Setup”

Choose a digital input to be used as a trigger for enabling freedrive.  Select that input on the left-hand window.

Set the “Action” to “Freedrive” from the drop-down menu

Wiring the Controller:

Recommended to use a normally open (NO) single-pole, single throw (SPST) switch or button.  While this switch is toggled on, the freedrive of the UR robot will be enabled.  Using a momentary or latching switch is a design decision.  For momentary switches, the switch must be held in the “closed” position, whereas a latching switch will stay in the “closed” position until the operator changes it back to the “open” position.

Wire a SPST button to the chosen digital input.  One lead going to the button should be attached to the 24VDC signal, and the other to the digital input.