NIST Collaborative Robotics Testbed

Standard Operating Procedure:  Human-Subject Testbed Setup and Adjustments

Version:  1.0

Last modified date:  10 January, 2025

Last modified author (name and contact email):  	Jeremy Marvel, jeremy.marvel@nist.gov

1	Overview	5

2	Before You Begin:  Safety	6

2.1	Related FLHRs	6

2.2	Required Training	6

2.3	Tool Storage	7

2.4	Eye Safety	7

2.4.1	Material Contact with Eyes	7

2.4.2	Laser Safety	7

2.5	Ear/Hearing Safety	7

2.6	Hand/Skin Safety:  Metal and Plastic Particulates and Shavings	7

2.7	Respiratory Safety	8

3	Activity:  Adding and Removing Parts to/from the Testbed	9

3.1	Process	9

3.2	Best Practices	9

3.3	Hazard Assessment	9

4	Activity:  Drilling, Cutting, and Grinding	11

4.1	Drilling:	11

4.1.1	Process	11

4.1.2	Best Practices	11

4.1.3	Hazard Assessment	11

4.2	Cutting:	11

4.2.1	Process	12

4.2.2	Best Practices	12

4.2.3	Hazard Assessment	12

4.3	Filing and Sanding:	12

4.3.1	Process	12

4.3.2	Best Practices	12

4.3.3	Hazard Assessment	13

5	Activity:  Manufacturing Component Parts	14

5.1	Process	14

5.2	Best Practices	14

5.3	Hazard Assessment	14

6	Activity:  Moving/Removing Component Parts	16

6.1	Moving the Robot:  UR3e (mass:  11.2 kg ~17.7 lb; reach 500 mm)	16

6.1.1	Process	16

6.1.2	Best Practices	17

6.1.3	Hazard Assessment	17

6.2	Robot:  Robotiq 2-finger adaptive gripper (0.925 kg)	17

6.2.1	Process	17

6.2.2	Best Practices	18

6.2.3	Hazard Assessment	18

7	Activity:  Configuring, Adjusting and Testing Robots	20

7.1	Robot:  UR3e (mass:  11.2 kg ~17.7 lb; reach 500 mm)	20

7.1.1	Process	21

7.1.2	Best Practices	21

7.1.3	Hazard Assessment	21

7.2	Robot:  Robotiq 2-finger adaptive gripper (0.925 kg)	21

7.2.1	Process	21

7.2.2	Best Practices	22

7.2.3	Hazard Assessment	22

8	Activity:  Electronics Work	24

8.1	Soldering	24

8.1.1	Process	24

8.1.2	Best Practices	24

8.1.3	Hazard Assessment	24

8.2	Cutting, Stripping and Crimping Wire	25

8.2.1	Process	25

8.2.2	Best Practices	25

8.2.3	Hazard Assessment	25

9	Activity:  Laser Measurement and Alignment	27

9.1	Process	27

9.2	Best Practices	27

9.3	Hazard Assessment	27

10	Activity:  Moving the Testbed	28

10.1	Process	28

10.2	Best Practices	28

10.3	Hazard Assessment	28

11	Emergency Response Plan	29

11.1	Event:  Fire	29

11.2	Material Hazards	29

11.2.1	Event: Contact with Hot Tools or Parts	29

11.2.2	Event: Tool Breaks or Loose Workpiece	29

11.2.3	Event: Removed Material Contact with Skin or Eye	29

11.2.4	Event: Removed Material Embedded in Skin or Eye	29

11.3	Chemical Hazards	30

11.3.1	Event:  Chemical Contact with Eyes	30

11.3.2	Event:  Chemical Contact with Skin	30

11.3.3	Event:  Chemical Spill	30

11.3.4	If the chemical agent is known	30

11.3.5	If the chemical agent is unknown	30

11.3.6	Event:  Chemical Contact with Eyes	30

11.3.7	Event:  Chemical Contact with Skin	30

11.3.8	Event:  Inhalation of Chemical Vapors	30

11.4	Building Related Hazards	31

11.4.1	Event: Fire	31

11.4.2	Event: Hazardous Weather	31

11.4.3	Event: Power Outage	31

Change Log:

3 October, 2024

- First draft of the SSOP

10 January, 2025

- Updated draft based on GL and DC feedback

# Overview

This document outlines the safe standard operating procedures for the setup, modification, and testing of the human subject studies testbed for the collection of data for human-robot interaction research.

This activity is limited to the Human-Robot Interaction laboratory on the second floor of Building 202.

The testbed will consist of a mobile cart with a mounting plate on top to support a small industrial robot, assembly taskboards, parts trays, sensors, and other ancillary equipment.  It is expected that all supporting and controlling equipment will be stored on a lower shelf of the cart, or potentially on a separate structure or cart.

Sensors that will be used to capture human subject data will be mounted on or near the testbed, and elevated such that they will not be obstructed by the robot or other equipment.  Testbed-mounted sensors will be attached via rigid structure connected (either via bolted mounting plates, or by clamps) to the surface of the testbed.

The construction process and related processes require the use of powered tools, some of which are considered hazardous to use.  Operators of these tools shall be trained in their use.  Training includes reading and demonstrating understanding of this document.

The standard operating procedures described in this document are provided to ensure the safety of activities involving constructing and modifying the human study testbed.  This document is intended to serve as both training, and as a reference guide for the handling of materials and equipment.  A copy of this document shall be kept present with the human study testbed for reference during setup and future modifications.

Access to the testbed and related components and tools is considered restricted to authorized personnel only.

# Before You Begin:  Safety

Be aware that the construction process includes the use of powered tools, which can cause injury if misused.  Proper safety precautions shall be taken.  The specific hazards associated with these tools and other related equipment are provided in the First Level Hazard Review (FLHR) repository (https://mmlweb.nist.gov/safety/).

## Related FLHRs

The following FLHRs are specifically identified for the following actiities, and are not covered in this SOP:

| Activity Name (ID) | Associated Activities | Link |
| --- | --- | --- |
| Use of Universal Robots manipulators with CB and E-Series controllers (735.13.0127) | Installing, modifying, and testing the testbed’s attached robot | https://mmlweb.nist.gov/safety/activities/12719 |
| Collaborative Robotics Laboratory Laser Cutter (735.13.0059) | Using the laser cutter to create components for the testbed | https://mmlweb.nist.gov/safety/activities/6464 |
| Material Machining Using a CNC Mill (735.13.0084) | Using the CNC mill to manufacture parts for the testbed | https://mmlweb.nist.gov/safety/activities/10328 |
| Resin-Based 3D Printing (735.13.0096) | Using resin-based 3D printers to create parts for the testbed | https://mmlweb.nist.gov/safety/activities/9707 |
| FDM-Style 3D Printing (735.13.0021) | Using filament-based 3D printers to create parts for the testbed | https://mmlweb.nist.gov/safety/activities/9992 |
| Use of hand tools, including power screwdrivers and drills (735.13.0120) | Using hand tools for attaching and removing components, including removing material to enable securely connecting components | https://mmlweb.nist.gov/safety/activities/12529 |
| Soldering and Electronic Assembly (735.13.0121) | Using soldering tools to create and modify electronic components, electrical connections, and custom printed circuit boards. | https://mmlweb.nist.gov/safety/activities/12530 |
| Class 2 visible laser use in PHRI laboratory (735.13.0125) | Using laser-based equipment to align sensors and measure distances for the testbed | https://mmlweb.nist.gov/safety/activities/12681 |

## Required Training

Personnel who are involved with this activity shall complete the following identified training courses.  Rationale for the requirement of training is also provided.

| Training | In-Person or Virtual | Rationale |
| --- | --- | --- |
| Ladder Safety 2.0 | Virtual on CLC | Installation of elevated sensors and equipment may require the use of ladders to reach positions not accessible from the ground. The maximum elevantion needed to be reached is 10 feet (~3.1m). |
| Electrical Safety 2.0 | Virtual on CLC | The testbed consists of a metal structure with powered equipment. Installing, wiring, and powering equipment may introduce electrical hazards. |
| Slips, Trips and Falls 2.0 | Virtual on CLC | During the construction and commissioning of the testbed, it is expected cables, loose materials, and ancillary equipment may be located on or slightly above the floor around the testbed itself, presenting slipping and tripping hazards. |
| Safe Manual Material Handling | Virtual on CLC | Activities may require heavy or large materials (e.g., ladders) to be moved on to, off of, or around the testbed, and in some cases the testbed itself may need to be moved or lifted. |

After the trainings are complete, the prospective authorized user shall send an email to the principal investigator (jeremy.marvel@nist.gov) with screenshots of the training certificates from CLC and a statement indicating that the Collaborative robot arm trainings were completed.

## Tool Storage

All tools, when not in use, shall be stored in the appropriate tool chests to ensure easy location, access, and protection against accidental activation or contact with personnel.  These tool chests include locks to prevent unauthorized access to tools with specific hazards (e.g., utility knives, drill bits, and power tools).

Locking tools in the tool chests to prevent unauthoried access/use is recommened, but not required, as additional activities in the laboratory may require their use.

## Eye Safety

### Material Contact with Eyes

Some tasks associated with this activity, such as cutting and drilling, may present hazards that could result in eye injury.  The use of eye protection is required for personnel working on or near the human subject testbed during setup and maintenance.  Eye protection includes the proper use of PPE such as safety glasses.

Proper PPE removal procedures shall be used.

Safety glasses shall be disposed of properly if damaged.

### Laser Safety

Some tasks may involve the use of class 2 lasers for alignment and measurement purposes, and manufacturing processes may involve a 25 watt class 3 laser cutter.  Care shall be taken to avoid accidental direct exposure of the eyes to the laser beams.  Lasers shall be turned off while not in active use, and the use of safety glasses that include laser protection is recommended.

Proper PPE removal procedures shall be used

Safety glasses shall be disposed of properly if damaged.

## Ear/Hearing Safety

The use of hand tools (e.g., hammers or power tools) may produce loud noises that could damage hearing.  Hearing protection is provided in the laboratory, and consists of both disposable ear plugs and reusable over-ear muffs.

Proper PPE removal proecdures shall be used.

Damaged hearing protection shall be disposed of properly.  Single-use ear plugs shall also be disposed of after use.

## Hand/Skin Safety:  Metal and Plastic Particulates and Shavings

Skin contact to materials removed from component parts of the testbed may result in small cuts, lacerations, punctures, or other injuries.  The use of skin protection is strongly recommended when using drills or cutting tools so long as the use of PPE does not introduce new hazards (e.g., protective gloves and clothing may become engaged with cutting surfaces of power tools, resulting in significant injury).  Skin protection includes the use of gloves (nitrile gloves are provided for general purpose use) and protective clothing (lab coats are provided for general purpose use).

Nitrile gloves shall be disposed of after use.

## Respiratory Safety

Tasks like cutting and grinding may generate particulates that may become airborne cause respiratory irritation.  Dust masks are provided and required when drilling, cutting, grinding and sanding using powered tools.

Respirators are not required.

# Activity:  Moving Parts to/from the Testbed

As a general task associated with constructing and commissioning the testbed, parts such as structural components, sensors, robotic manipulators, electronics, and safeguards will be periodically moved to and from the testbed setup.

## Process

This activity involves the indefinite repetition of activities of moving, installing, and modifying parts and equipment.

- Plan the lift and transport actions prior to starting the material handling process

- Inspect the materials to be moved to ensure that they can be safely transported.  In particular, look for and mitigate the following hazards:

  - Sharp corners or edges that could present laceration risks

    - Cover any sharp features that could expose personnel to hazards,

    - Wear proper PPE such as long-sleeved shirts, pants, and cut-resistent gloves,

    - Place the object into a container that can support the bulk of the objects/materials being moved, or

    - Use a cart to transport parts/materials

  - Loose components that could fall during transport, causing impact injuries or tripping hazards

    - Secure all loose parts prior to lifting and/or moving, or

    - Place the object into a container that cansupport the bulk of the objects/materials being moved

  - Heavy objects that could cause strain injuries while lifting/transporting, or blunt force injuries if accidentally dropped

    - Have two or more people move the object together if it is beyond your capacity to lift alone,

    - Use a cart to transport parts/materials, or

    - Consider dismantling large or very heavy objects to reduce weight

- Inspect the path to be taken, and ensure that it is sufficiently wide to accommodate the passage of personnel and equipment, and is free of obstructions and slipping/tripping hazards

- Inspect the destination location, and ensure that it is sufficiently clear to accommodate the placement and, if necessary, installation of equipment and materials

- Disconnect any cables, hoses, or other attached components from the object to be moved

- If the object to be moved is secured to a surface using bolts, adhesives, or other mechanical means, ensure that the object is properly detached prior to attempting to move

- If picking up the object from the ground, bend your knees to crouch to a postion from which the object(s) can be safely lifted

- Keeping your back straight, use your arm and leg muscles to lift the object, keeping it close to your body

## Best Practices

- Whenever possible, work in teams of two or more to assure assistance is available for moving large/heavy objects, and that proper responses to hazards or injuries can be quickly achieved

## Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Pinching/crushing |  |  |  |  |  | 1 |
| Impact (dropped parts) |  |  |  |  |  |  |
| Ergonomics (strain) |  |  |  |  |  |  |
| Ergonomics (extension) |  |  |  |  |  |  |
| Slips trips falls |  |  |  |  |  |  |

# Activity:  Drilling, Cutting, and Grinding

General description of this activity

| Related Equipment | Associated Hazards | FLHR |
| --- | --- | --- |
| Hand tools: Hand saws, utility knives, files, sandpaper | Skin/eye injury Skin/eye irritation Respiratory injury | Use of hand tools, including power screwdrivers and drills (735.13.0120.102224i) |
| Power tools: Cordless drills, rotary tools, orbital sanders | Skin/eye injury Skin/eye irritation Respiratory injury | Use of hand tools, including power screwdrivers and drills (735.13.0120.102224i) |

Always wear appropriate eye protection when using drills, cutting tools (hand saws, rotary tools, utility knives, etc.), and grinding tools (orbital sanders, rasps, rotary tools, sand paper, etc.).

## Drilling:

General description of the drilling activity

### Process

### Best Practices

- Always keep hands and fingers away from the drill bit.

- Properly secure all loose parts/components before attempting to drill materials.

- Clean up any material waste by vacuuming any loose particulates from the floor and work surface.

### Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Skin/eye injury from drill and rotary tool |  |  |  |  |  |  |
| Skin/eye irritation from drill and rotary tool |  |  |  |  |  |  |
| Respiratory injury from drill and rotary tool |  |  |  |  |  |  |

## Cutting:

General description

### Process

### Best Practices

- Always cut away from yourself and others.

- Properly secure all loose parts/components before attempting to cut materials.

- Keep limbs and loose clothing away from the cutting surface.

- Clean up any material waste by throwing large waste pieces into the trash, and vacuuming any loose particulates from the floor and work surface.

### Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Skin/eye injury from hand tool |  |  |  |  |  |  |
| Skin/eye irritation from hand tool |  |  |  |  |  |  |
| Respiratory injury from hand tool |  |  |  |  |  |  |
| Skin/eye injury from rotary tool |  |  |  |  |  |  |
| Skin/eye irritation from rotary tool |  |  |  |  |  |  |
| Respiratory injury from rotoary tool |  |  |  |  |  |  |

## Filing and Sanding:

General description

### Process

### Best Practices

  - Always remove material away from yourself and others.

  - Properly secure all loose parts/components before attempting to grind surfaces.

  - Keep limbs and loose clothing away from the grinding surface.

  - Do not use filing tools near flammable materials, as metal parts may eject hot sparks that could ignite and create a fire hazard.

  - Clean up any material waste by vacuuming any loose particulates from the floor and work surface.

### Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Skin/eye injury from hand tool |  |  |  |  |  |  |
| Skin/eye irritation from hand tool |  |  |  |  |  |  |
| Respiratory injury from hand tool |  |  |  |  |  |  |
| Skin/eye injury from power tool |  |  |  |  |  |  |
| Skin/eye irritation from power tool |  |  |  |  |  |  |
| Respiratory injury from power tool |  |  |  |  |  |  |

# Activity:  Manufacturing Component Parts

Certain fixtures, structural components, and other parts may need to be custom built to meet the needs of the testbed.  For component parts that cannot be manufactured by NIST services or outside contracts, these parts will be manufactured by project staff using 3D printing, laser cutting, and CNC milling.

| Related Equipment | Associated Hazards | FLHR |
| --- | --- | --- |
| 3D printer | Burn risk Fire risk Toxic chemicals Skin injury | FDM-Style 3D Printing (735.13.0021) Resin-Based 3D Printing (735.13.0096) |
| Laser cutter | Fire risk Skin injury | Collaborative Robotics Laboratory Laser Cutter (735.13.0059) |
| CNC mill | Skin/eye injury Skin/eye irritation | Material Machining Using a CNC Mill (735.13.0084) |

Refer to the above FLHR(s) for SOPs involving the laser cutter, CNC mill, and 3D printers.

All other manufacturing procsses will be added to this section when they are identified and defined.

## Process

## Best Practices

## Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Burn risk from 3d printer |  |  |  |  |  |  |
| Fire risk from 3d printer |  |  |  |  |  |  |
| Toxic chemicals from resin 3d printer |  |  |  |  |  |  |
| Skin injury from 3d printer |  |  |  |  |  |  |
| Fire risk from laser cutter |  |  |  |  |  |  |
| Skin injury from laser cutter |  |  |  |  |  |  |
| Skin/eye injury from mill |  |  |  |  |  |  |
| Skin/eye irritation from mill |  |  |  |  |  |  |

# Activity:  Installing and Attaching Component Parts

General description

| Related Equipment | Associated Hazards | FLHR |
| --- | --- | --- |
| Hand tools: Hand saws, utility knives, files, sandpaper | Skin/eye injury Skin/eye irritation Respiratory injury | Use of hand tools, including power screwdrivers and drills (735.13.0120.102224i) |
| Collaborative robot | Struck-by hazards Crushing/pinching hazards | Collaborative Robotics Research (735.13.0001) |

When component parts are to be added to or removed from the testbed, the activity may involve handling parts that are heavy or sharp, and may therefore present pinching, cutting, or scraping hazards.  Occasionally, cutting or striking tools may be required to separate parts from the testbed.  During the removal process, nearby, unsecured parts may fall or be ejected from the testbed surface, presenting hazards to lower extremities.  Always ensure a clean working surface around the source and destination of the parts being handled.

Specific equipment may require detailed instructions for proper handling when installing, moving, or removing.  These details are provided in this section.

## Installing the Robot:  UR3e (mass:  11.2 kg ~17.7 lb; reach 500 mm)

General description

### Process

#### Attaching the robot to the testbed

#### Connecting the robot to the controller

#### Installing external safeguards

#### Powering the controller

#### Initializing the robot

E-series robot has minimum motion during initialization, so no hazard exists

### Best Practices

- The robot and its controller shall be disconnected from power sources (including electrical and pneumatic) before attempting to move.  The robot shall be disconnected from the robot controller before attempting to move.

- The robot shall be properly stored when not actively being used.  Proper storage includes:  1) a mounting plate secured to a heavy and stable work surface, and 2) the original packaging in which the robot was shipped.

- Before moving, ensure the proper storage solution is selected, installed correctly, and is free from nearby debris or obstructions.  The travel path from the robot’s current location to its destination shall also be free from debris and obstructions.

- It is recommended, but not required, that two people work together to move the robot to ensure proper management of connected components such as ancillary equipment, tooling, and cables/hoses.

- If the robot is currently mounted to a plate, carefully remove the bolts securing the robot to the surface.  The robot must be supported during this task, as it may be in an unstable configuration that could result in the robot falling when the securing bolts are removed.  Supporting the robot can be done with a single hand given its low mass.

### Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Pinching/crushing |  |  |  |  |  | 1 |
| Ergonomics (strain) |  |  |  |  |  |  |
| Ergonomics (extension) |  |  |  |  |  |  |
| Ergonomics (repetitive strain) | Screwing things in |  |  |  |  |  |
| Electric shock |  |  |  |  |  |  |
| Electric fire |  |  |  |  |  |  |

## Robot:  Robotiq 2-finger adaptive gripper (0.925 kg)

General description

### Process

#### Mounting the robot gripper

#### Mounting the gripper fingers

### Best Practices

- The robotic gripper attaches to the tool flange of the robot using four retaining bolts.  During installation and removal, use one hand to support the gripper while using the other to loosen and remove the bolts.

- The robotic gripper has a rubberized signal cable that attaches to the robot arm’s tool-flange I/O port.  The gripper came with a plastic gasket that goes over the I/O port of the robot arm, and prevents accidental contact with the signal cable during operation.

- The robotic gripper may remain attached to the robot while moving the robot.

- The stock metal fingers have been replaced with 3D printed finger tips. If these finger tips break, carefully remove the retaining bolt from the finger tip and replace the damaged part with a new, undammaged finger tip.

### Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Pinching |  |  |  |  |  | 1 |
| Ergonomics (extension) |  |  |  |  |  |  |

# Activity:  Configuring, Adjusting and Testing Robots

General description

| Related Equipment | Associated Hazards | FLHR |
| --- | --- | --- |
| Collaborative robot | Struck-by hazards Crushing/pinching hazards | Collaborative Robotics Research (735.13.0001) |

This activity is intended to verify functionality and performance of robotic equipment.  Tasks include jogging the robot using the teach pendant and free-drive (i.e., using a gravity compensation mode to allow the user to directly move the robot’s joints by pushing or pulling on the robot), opening and closing the attached gripper, and adjusting and verifying software limits on robot motions.

The activity may involve handling parts that are heavy and blunt, and may therefore present pinching and struck-by hazards.  During this activity, parts that are held or manipulated by the robot may fall or be ejecte from the testbed, presenting struck-by hazards to lower extremities.  Always ensure a clean working surface around the source and destination of the parts being handled.

Specific equipment may require detailed instructions for proper handling when installing, moving, or removing.  These details are provided in this section.

## Robot:  UR3e (mass:  11.2 kg ~17.7 lb; reach 500 mm)

The testbed surface has been specifically designed to accommodate a UR3e industrial robot, with threaded screw holes for mounting and a through-hole for cable access to the controller below the table surface.  Thus the position of the UR3 is known a priori.  However, as installed, it is not registered to the tabletop, any sensors, or the workpieces to be used during the human subject assembly activity.  A critical component of this is to align the major X and Y axes of the robot’s motions with the X and Y axes of the testbed surface.  This section will describe the steps necessary to accommodate the configuration and initial testing of the robotic manipulator.

### Process

#### Configure the robot installation

#### Testing the alignment with the testbed

### Best Practices

- The robot shall be securely connected to the testbed prior to commanding motions.  Additionally, the robot shall be properly configured with the correct orientation (for gravity compensation) and end-of-arm tooling mass.

- The robot shall be properly stowed when not actively being used.  Proper stowage includes:  1) moved to a position where all parts of the robot (including any held objects) are contained within the boundaries defined by the horizontal edges of the testbed surface, and 2) drive power is disabled to prevent unexpected motion.

- Before commanding robot motions, ensure the work area around the robot is free of debris or obstructions.  The travel path from the robot’s current location to its destination shall also be free from debris and obstructions.

- At least one person shall be stationed by an external emergency stop, and shall be prepared to activate emergency stops in the event of a hazardous event.

- The robot’s motions shall be limited to a velocity that the operator feels comfortable with.  During jogging motions, this velocity is typically capped at 0.5m/s, but velocities may go as high as 2.0m/s during program execution.

### Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Pinching/crushing |  |  |  |  |  | 1 |
| Impact |  |  |  |  |  |  |

## Robot:  Robotiq 2-finger adaptive gripper (0.925 kg)

General description

### Process

#### Install the controller software on the e-series controller

#### Configure the gripper

Testing the gripper

### Best Practices

- The robotic gripper shall be securely connected to the robot’s tool flange prior to commanding motions.

- The robot gripper receives its power from the robot, but may be independently disabled using the robot’s teach pendant.  Initializing the gripper will cause the gripper to open and close its grasp.  It is recommended the gripper be left in an “open” configuration prior to disabling the gripper and/or shutting off the robot.  This will prevent any held objects from being dropped or ejected when the gripper is enabled.

- If the gripper finger tips are damaged or broken, immediately stop all tasks using the robot and replace the affected finger tips.

### Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Pinching/ crushing hazard from opening and closing the gripper during testing | UR controller actuating the robotic gripper | Significant injury if operator’s fingers are in the mechanical works of the gripper | Very unlikely (has not happened to our knowledge, and there is no legitimate reason to stick one’s fingers into the gripper mechanics) | Engineering: - Gripper’s mechanism is designed to leave a large gap, preventing pinching/ crusthing hazard - Gripper’s fingers are designed to break with minimal force Administrative: - Safe practices, including the review and demonstrated understanding of the SOP. | Improbable | 1 |

# Activity:  Electronics Work

General description

| Related Equipment | Associated Hazards | FLHR |
| --- | --- | --- |
| Soldering iron Desoldering tool | Burn risk Fire risk Skin injury Skin/eye irritation Respiratory irritation | Soldering and Electronic Assembly (735.13.0121) |

All work involving the construction or modification of electronic components shall be conducted at the electronics workbench at the end of the room.  The electronics workbench has the appropriate tools and safety equipment to ensure personnel safety against risks of electric shock, and hazards associated with soldering processes.

When constructing or servicing electronic components, all power sources must be disconnected and/or discharged prior to working with the equipment to eliminate risk of electric shock.  Personnel shall also use appropriate electrostatic discharge (ESD) equipment to prevent damage to electronic coponents.

Specific equipment may require detailed instructions for proper execution of electronics tasks.  These details are provided in this section.

## Soldering

General description

### Process

### Best Practices

- Any activity involving melting solder (e.g., soldering components, or using a heat gun to reflow solder) requires the use of a desktop fume extractor.  Fume extractors are already installed and powered at the electronics workbench.

- The soldering irons in the human-robot interaction lab have adjustable heat settings.  Set the soldering iron temperature to the lowest setting required to melt the solder.

- To avoid burn hazards, use clamps or helping hands to hold parts being soldered, and allow the parts to cool for several minutes before attempting to handle with bare hands.

### Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Burns |  |  |  |  |  | 1 |
| Fires |  |  |  |  |  |  |
| Ergonomics (repetitive stress) |  |  |  |  |  |  |

## Cutting, Stripping and Crimping Wire

General description

### Process

### Best Practices

- Always use wire cutters to cut wire and cable.  Do not use scissors or knives.

- Always use wire strippers to remove the protective sheathing from wire and cables.  Do not use a knife.

- Always use a dedicated wire crimper to affix connectors, plugs, and attachments.  Do not use pliers, tweezers, hemostats, or other grasping tools.

### Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Cuts |  |  |  |  |  |  |
| Ergonomics (repetive stress) |  |  |  |  |  |  |

# Activity:  Laser Measurement and Alignment

General description

| Related Equipment | Associated Hazards | FLHR |
| --- | --- | --- |
| Laser pointer | Non-ionizing radiation, Eye injury | Class 2 visible laser use in PHRI laboratory (735.13.0125) |
| Laser range tool | Non-ionizing radiation, Eye injury | Class 2 visible laser use in PHRI laboratory (735.13.0125) |

Laser equipment may be used for measurement (e.g., distance measures) and alignment (e.g., laser pointers) purposes.  Only class 1 (eye safe) or class 2 laser equipment may be used during testbed setup and adjustment activities.  Lasers should be energized only as long as they are actively being used, and then turned off when not in use.

## Process

## Best Practices

## Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Eye injury |  |  |  |  |  | 1 |

# Activity:  Moving the Testbed

General description

| Related Equipment | Associated Hazards | FLHR |
| --- | --- | --- |
| None | N/A | N/A |

When changing the location of the testbed within the human-robot interaction laboratory, loose objects may fall from testbed surface and become tripping hazards.  The robot, controllers, and all ancillary equipment and sensors shall be rigidly affixed to the testbed—or removed entirely for transport separately—prior to attempting to move the testbed.

## Process

## Best Practices

## Hazard Assessment

| Specific Hazard (Explanation) | Hazard Source | Severity | Occurrence Likelihood | Controls | Controlled Likelihood (Explanation) | Controlled RHI |
| --- | --- | --- | --- | --- | --- | --- |
| Crushing (Rolling) |  |  |  |  |  | 1 |
| Slips/trips |  |  |  |  |  |  |
| Inpact wit testbed and onboard equipment |  |  |  |  |  |  |
| Impact with things falling off |  |  |  |  |  |  |

# Emergency Response Plan

## Event:  Fire

In the event of fire, immediately contact NIST Safety at x2222 to report the fire.

Authorized personnel may use an approved ABC fire extinguisher (combustible materials, flammable liquids, and flammable gases) to attempt to extinguish the fire.  Acceptable fire extinguisher types include dry powder, alcohol-resistant foams, and carbon dioxide.

Do not use water to combat fires.  Water may spread the flames.

## Material Hazards

### Event: Contact with Hot Tools or Parts

Cutting, drilling and grinding processes generate heat, and certain materials may retain a high temperature after cutting for several minutes.  Should accidental contact occur, immediately check all impacted personnel for potential injuries such as cuts or burns, contacting NIST Safety at x2222 if there has been an injury.

### Event: Tool Breaks or Loose Workpiece

Operations involving powered tools may result in cutting implements breaking or workpieces being forcibly ejected.  If such events occur, recover any lost pieces of the damaged tools or parts and properly dispose of them; these pieces may have extremely sharp edges/points, so proper PPE is required when handling broken components.

Contact NIST Safety at x2222 if there has been an injury.

### Event: Removed Material Contact with Skin or Eye

The removed material from the workpiece—and the workpiece, itself—are likely to have extremely sharp edges, and may result in skin or eye damage if contacted directly.

If contact with skin, immediately wash hands with warm water and soap and flush the affected region of skin with warm water. Do not brush or rub the affected region, as it may drive small slivers into the skin.

In contact with eye, immediately wash hands with warm water and soap. Flush eyes with gently-flowing

water immediately for several minutes. Remove contact lenses, if present and easy to do.

DO NOT USE AN EYE WASH, as the high flow of water may cause chipped material to imbed in the eye and surrounding soft tissue. Seek medical attention to ensure that all material has been removed from the eye. This can be preformed by the NIST Health Unit during normal business hours.

### Event: Removed Material Embedded in Skin or Eye

If removed materials are embedded in the skin, immediately wash hands with warm water and soap. Flush the affected region with warm water. Do not brush or rub the affected region, as it may further embed the material. If possible, gently remove the material from the skin.

If removed materials are embedded in the eye, immediately contact NIST Safety at x2222 and/or report to the Heath Unit in building 101. DO NOT TOUCH OR RUB EYE, OR ATTEMPT TO REMOVE THE OBJECT.

## Chemical Hazards

This activity does not require the regular use of chemicals or materials with chemical hazards.  However, some tasks may include the use of chemicals that present specific hazards.

### Event:  Chemical Contact with Eyes

Flush eyes immediately for several minutes using the eye wash station next to the fume hood or a sink in a nearby restroom.  Remove contact lenses, if present and easy to do.  Continue rinsing.

When possible, report to the Heath Unit in building 101 for a full evaluation.

### Event:  Chemical Contact with Skin

Remove any contaminated clothing immediately, and rinse skin in the closest restroom for several minutes.

When possible, report to the Heath Unit in building 101 for a full evaluation.  Be prepared to describe chemicals used and their quantities/concentrations.

### Event:  Chemical Spill

In the event of a chemical spill, use the following steps to eliminate the hazard.

### If the chemical agent is known

Always wear appropriate PPE.

Use the clean-up instructions specified in the agent’s SDS.

### If the chemical agent is unknown

Always wear appropriate PPE.

Use the spill kit located to the left of the fume hood.

### Event:  Chemical Contact with Eyes

Flush eyes immediately for several minutes using the eye wash station next to the fume hood or a sink in a nearby restroom.  Remove contact lenses, if present and easy to do.  Continue rinsing.

When possible, report to the Heath Unit in building 101 for a full evaluation.

### Event:  Chemical Contact with Skin

Remove any contaminated clothing immediately, and rinse skin in the closest restroom for several minutes.

When possible, report to the Heath Unit in building 101 for a full evaluation.  Be prepared to describe chemicals used and their quantities/concentrations.

### Event:  Inhalation of Chemical Vapors

The inhalation of chemical vapors may result in symptoms such as:

- Coughing,

- Shortness of breath,

- Eye/nose/throat irritation,

- Headache, and

- Dizziness.

If any of these symptoms manifest, immediately leave the laboratory and breathe fresh air.  If others are in the laboratory, advise them to do the same.

When recovered, contact NIST Safety at x2222, and report the circumstances under which you were operating when the exposure occurred.  Be prepared to describe the chemicals used and their quantities/concentrations.

When possible, report to the Heath Unit in building 101 for a full evaluation.  Be prepared to describe chemicals used and their quantities/concentrations, and any symptoms experienced.

Do not return to the laboratory until breathing fresh air for a few hours, and having been given the all-clear by emergency personnel.  Avoid closed spaces with poor air flow.

## Building Related Hazards

### Event: Fire

If a fire event occurs in the building and the building fire alarm sounds, immediately turn off any powered equipment, place any tools/parts on the floor in a safe and stable position, and then evacuate the building per NIST guidelines for building evacuation.

### Event: Hazardous Weather

If a hazardous weather event occurs and the building alarm sounds, immediately turn off any powered equipment, place any tools/parts on the floor in a safe and stable position, and then move all personnel to the shelter-in-place zone in the building in a safe and orderly fashion.

### Event: Power Outage

Activate the E-stop button for any robotic equipment that was previously powered, and turn off the power to any equipment operating under backup or battery power.

Move to a safe location away from any hazards until power has been restored.